SPCE 5045

Homework #2

60 points total

NOTICE: The charts you will need, Figure 7-11 and 7-12, require a very close look at the data curves AND the scales.

Chapter 7:

1. A satellite is designed to operate in a circular polar orbit at an altitude of 1000 km.  A large, sensitive electronics subsystem on the satellite must be shielded against radiation.  An area of 1.2 E5 cm2 must be shielded with a sufficient thickness of aluminum to reduce the radiation dosage to less than 104 rads/year.

a) (10 points) What is the required shielding thickness?

b) (10 points) What is the total weight of this shielding?

2. (10 points) A geosynchronous satellite (altitude approximately 35,800 km) is required to operate for 10 years in a peacetime environment. The solid-state electronic devices on-board are surrounded by 100 mils (0.1 inch) of aluminum.  What is the total radiation dosage that the electronics must tolerate?

3. (10 points) What is “ESD”? Describe how parts of a spacecraft might be a source of ESD and what can be used to reduce this?

4. (10 points) What approaches to spacecraft design/operation can be used to minimize single event upsets (SEUs)?

5. (10 points) What is “outgassing”? Why is this an important consideration for spacecraft design?